# KRAI 2026 — Button Mapping

Master Board (ESP32-S3) — Controller PS4 DualShock 4

# 1. Joystick & DPAD

| Input | Tanpa modifier | + R2 hold | + Circle hold |
| --- | --- | --- | --- |
| Analog kiri (LX/LY) | Motion vx/vy (ANALOG mode) | Armbox motor Y jog (DPAD mode) | Armbox FB toggle (DPAD mode) |
| Analog kanan (RX/RY) | Yaw ±1° | Motor X/Y jog (gripper) | — |
| DPAD | Motion vx/vy (DPAD mode) | Armbox motor Y jog (ANALOG mode) | Armbox FB toggle (ANALOG mode) |

# 2. Tombol — Mode Manual (mode = 0)

| Tombol | Edge/Hold | Aksi |
| --- | --- | --- |
| Cross | — | Kosong |
| Circle | Hold + arah (edge) | Arm box manual: Kiri→FB toggle R Kanan→FB toggle L Atas→done L Bawah→done R |
| Square | Hold | Freeze motion (vx=0, vy=0) |
| Triangle | Hold | Servo B manual (hanya saat READY_TO_STAB) |
| L1 | Hold | Slow — yaw step 150ms, gripper step 5 |
| R1 | Hold | Fast — yaw step 25ms, gripper step 40 |
| L2 + Triangle | Edge | User-defined action (gripper_control) |
| L2 + Circle + analog kiri ke kanan | Hold | Arm box R: pne ON → ARMBOX_WAIT |
| L2 + Circle + analog kiri ke kiri | Hold | Arm box L: pne ON → ARMBOX_WAIT |
| L2 + Square | Edge | User-defined action (armbox_control) |
| L2 + analog kanan | Hold | Yaw snap kardinal (0°/90°/180°/-90°) |
| L2 + R2 | Analog ≥250 | Flash lamp fire |
| R2 | Hold | Gripper mode — motor X/Y (analog kanan) + armbox Y jog (analog kiri/DPAD) |
| OPTIONS | Edge (zone 1) | SetupZone1 — servo homing + motor Y level 0 + motor X enc 200 (goto odom hanya jika mode=1) |
| OPTIONS | Edge (zone 2) | Toggle modeKinematics (goto ↔ kn) |

# 3. Tombol — Mode Otomatis (mode = 1)

| Tombol | Edge/Hold | Aksi |
| --- | --- | --- |
| Cross | — | Kosong |
| Circle | Hold + arah (edge) | Arm box manual: Kiri→FB toggle R Kanan→FB toggle L Atas→done L Bawah→done R |
| Square | Hold | Freeze motion (vx=0, vy=0) |
| Triangle | Hold | Servo B manual (hanya saat READY_TO_STAB) |
| L1 | Hold | Slow — yaw step 150ms, gripper step 5 |
| R1 | Hold | Fast — yaw step 25ms, gripper step 40 |
| L2 + analog kanan | Hold | Yaw snap kardinal (0°/90°/180°/-90°) |
| L2 + R2 | Analog ≥250 | Flash lamp fire |
| R2 | Hold | Gripper mode — motor X/Y (analog kanan) + armbox Y jog (analog kiri/DPAD) |
| OPTIONS | Edge (zone 1) | SetupZone1 — servo homing + odomGoto(1) + motor Y level 0 + motor X enc 200 |
| OPTIONS | Edge (zone 2) | Toggle modeKinematics (goto ↔ kn) |

# 4. Input Mode & Misc

| Tombol | Edge/Hold | Aksi |
| --- | --- | --- |
| SHARE | Edge | Toggle input mode (DPAD ↔ ANALOG) (diblok saat odom record ON) |
| L1+R1+L2+R2 | Edge | Toggle invert input (atas↔bawah, kiri↔kanan) |
| PS | — | Kosong |

# 5. Odom Record

| Tombol | Aksi |
| --- | --- |
| R3 + L3 | Toggle mode record ON/OFF (press, tanpa hold) |

## Odom Record Combos (mode record ON)

| Combo | Target |
| --- | --- |
| R1 + TOUCHPAD | Zone1 slot 0 |
| L1 + TOUCHPAD | Zone1 slot 1 |
| R2 + TOUCHPAD | Zone1 slot 2 |
| L2 + TOUCHPAD | Zone1 slot 3 |
| R1 + Triangle | Approach slot 0 |
| L1 + Square | Approach slot 1 |
| R2 + Triangle | Approach slot 2 |
| L2 + Square | Approach slot 3 |
| Cross + TOUCHPAD | Forest WP 2 |
| Square + TOUCHPAD | Forest WP 6 |
| Circle + TOUCHPAD | Forest WP 7 |
| Triangle + TOUCHPAD | Forest WP 11 |

# 6. Controller Mode (hardware)

| Hardware | Aksi |
| --- | --- |
| BOOT button (controller) — double-press 200ms | Toggle mode 0 (manual) ↔ 1 (auto) LED: putih=auto, coklat=manual Tersimpan ke NVS |
| BOOT button (master) — double-press 200ms | Toggle alliance RED ↔ BLUE LED: merah=RED, biru=BLUE Tersimpan ke NVS |

# 7. Speed & Interval

| Mode | Yaw step | Send interval | Gripper step |
| --- | --- | --- | --- |
| Normal | 50ms | 20ms | 15 enc |
| L1 (Slow) | 150ms | 20ms | 5 enc |
| R1 (Fast) | 25ms | 20ms | 40 enc |

# 8. Mode Behavior

| Mode | R2 OFF | R2 ON |
| --- | --- | --- |
| DPAD | DPAD → motion vx/vy L-stick → — R-stick → yaw | DPAD → armbox Y jog L-stick → armbox Y jog R-stick → motor X/Y |
| ANALOG | L-stick → motion vx/vy DPAD → — R-stick → yaw | L-stick → motion vx/vy DPAD → armbox Y jog R-stick → motor X/Y |

# 9. Auto (tanpa tombol)

| Trigger | Aksi |
| --- | --- |
| Proximity R (slave2) | Arm box R: pne ON → motor Y level 4 + motor X -255 → grab |
| Proximity L (slave2) | Arm box L: pne ON → motor Y level 4 + motor K -255 → grab |
| Motor Y sampai target | Hold PWM 50 (anti-gravitasi) + active=false |
| Motor K kena limit | Auto stop via motor k 0 |
| Proximity gripper (auto) | IDLE → CLOSING → UP → STRAIGHTEN → READY_TO_STAB |

# 10. Motor Y Level (default)

| Level | Encoder pulse |
| --- | --- |
| 0 | 0 |
| 1 | 300 |
| 2 | 600 |
| 3 | 900 |
| 4 | 1200 |
| 5 | 1500 |